Measurements and how am I going to measure them?

- IMU:

  - Linear acceleration (x,y,y components)

  - Angular acceleration (x,y,x coms)

  - Sampling rate: 100Hz

Specific IMU Options:

| Name | Cost | URL | Notes |
| --- | --- | --- | --- |
| LSM9DS1 | R148.00 | https://www.robotics.org.za/GY-LSM9DS1?search=IMU |  |
|  |  |  | No antenna |

- GPS/GNNS

  - Location (lat/long)

  - UTC timestamp

  - Sampling rate: 1Hz

  - Sampling medium: UART

Specific GPS/GNNS Options

| Name | Cost | URL | Notes |
| --- | --- | --- | --- |
| NEO-6M GPS Module | R110 +44 (antenna) | https://make.net.za/product/neo-6m-gps-module/?srsltid=AfmBOooCx9iijbeLUpBgI4jo7ulaTxZkBj_PMH9HLB6gIXKH7yQOn82f | Not in websites steven said 26.3mm x 35.5mm x 3.6mm Antenna: 25x25x4.6mm |
| PNT-SG3FS-TR | R186.96 | https://www.mouser.co.za/ProductDetail/Lantronix/PNT-SG3FS-TR?qs=amGC7iS6iy80vQnhNJGX0w%3D%3D | No antenna |

- Environmental Data

  - Air temperature, Humidity, Barometer

  - Sampling rate: 0.1Hz

  - Sampling Method: SPI/I2C

- MCU

  - Either Interrupt Driven or DMA

  - SPI or I2C

Specific MCU Options:

| Name | Cost | URL | Notes |
| --- | --- | --- | --- |
| STM32L476JGY6TR | R142,57 | https://www.digikey.co.za/en/products/detail/stmicroelectronics/STM32L476JGY6TR/6167042 |  |
| STM32U535NEY6QTR | R100,14 | https://www.digikey.com/en/products/detail/stmicroelectronics/STM32U535NEY6QTR/21349035 |  |
| STM32F411 Black Pill | R138,00 | https://www.robotics.org.za/STM32F411CEU6-MOD?search=stm32 | Breadboardable |

SIREN — Sensing Subsystem Design Decisions

===========================================

Author: Maarij

Course: EEE4113F Engineering Design 2026

Last updated: May 2026

---

1. MICROCONTROLLER

------------------

Selected: STM32F411CE (Black Pill development board)

Rationale:

- Cortex-M4 core with FPU sufficient for sensor polling and

SD card write operations at required data rates

- 512KB flash and 128KB SRAM adequate for firmware and

circular buffer implementation

- 5x SPI, 3x USART, 3x I2C peripherals cover all sensor

communication requirements without conflict

- DMA support on SPI and USART eliminates CPU blocking

during high-frequency IMU sampling

- Low cost (R138) within project budget constraints

- Arduino IDE compatibility via STM32duino board support

package accelerates development timeline

- USB DFU bootloader allows firmware flashing without

dedicated programmer hardware

Development environment: Arduino IDE 2.x with STM32duino

Upload method: STM32CubeProgrammer via SWD (ST-Link V2)

---

2. INERTIAL MEASUREMENT UNIT

------------------------------

Selected: LSM9DS1 (9-DoF MEMS IMU)

Interface: SPI (not I2C)

Sampling rate: 100 Hz

Data output: Raw 6-DoF — 3-axis acceleration (m/s²),

3-axis angular velocity (deg/s)

Rationale:

- 6-DoF motion capture sufficient to characterise full

ice floe dynamics including wave-induced heave, surge,

sway, pitch, roll and yaw

- 100Hz sampling rate resolves wave frequencies up to

0.5Hz (Nyquist) and captures transient high-frequency

events such as floe-floe collisions consistent with

SHARC V3.0 methodology (Noyce, 2024)

- SPI selected over I2C following documented I2C DMA

failures on STM32 in SHARC V3.0 (Noyce, 2024) — SPI

DMA on STM32 is deterministic and reliable

- Raw data transmitted without onboard processing to

preserve complete high-frequency dataset, consistent

with Herbers et al. (2012) IMU-GPS fusion methodology

Magnetometer status: DISABLED

- LSM9DS1 includes a 3-axis magnetometer which is

disabled in firmware

- Magnetometers are unreliable near the Antarctic

magnetic poles due to extreme magnetic interference,

as confirmed by SHARC V3.0 findings (Noyce, 2024)

and project supervisor guidance

- Only accelerometer and gyroscope data is logged

---

3. GNSS / GPS RECEIVER

-----------------------

Selected: u-blox NEO-6M

Interface: UART

Sampling rate: 1 Hz (continuous operation)

Data output: Latitude, longitude, UTC timestamp

parsed from NMEA $GPRMC sentences

Rationale:

- GPS position fixes at 1Hz exceed the minimum

15-minute sampling interval validated by Alberello

et al. (2020) for accurate Antarctic ice drift tracking

- Continuous operation mode adopted following SHARC V3.0

finding that cold/warm start failures in extreme

temperatures cause unacceptable lock-on delays (Noyce, 2024)

- UART DMA with idle-line interrupt minimises CPU load

during continuous NMEA sentence reception

Known limitation:

- NEO-6M is GPS-only (single constellation) versus

newer multi-constellation receivers (GPS, GLONASS,

Galileo, BeiDou)

- GPS satellite orbital inclination of 55 degrees

causes Dilution of Precision (DOP) degradation at

Antarctic latitudes (Hunt, 2020)

- Accepted as a known constraint given budget limitations

- Prototype demonstration in Cape Town confirmed

sufficient by project supervisor

- A multi-constellation receiver (e.g. NEO-M9N) is

recommended for any future field deployment

---

4. ENVIRONMENTAL SENSING

--------------------------

Status: DESCOPED from final prototype

Rationale:

- BME280 (temperature, pressure, humidity) was initially

specified based on literature gap identified in

Alberello et al. (2020) and Roach et al. (2025)

- Descoped to reduce firmware complexity and focus

development effort on primary motion and position

sensing objectives within project timeline

- Environmental sensing remains a recommended addition

for future iterations of the platform

---

5. DATA STORAGE

----------------

Medium: MicroSD card (8GB)

Interface: SPI (dedicated SPI peripheral, separate from IMU)

Library: SD.h (Arduino built-in via FatFS)

File format: CSV

CSV column structure:

timestamp_ms, ax, ay, az, gx, gy, gz, lat, lon

Field definitions:

timestamp_ms  — milliseconds since boot (uint32)

ax, ay, az    — linear acceleration in x, y, z (float, m/s²)

gx, gy, gz    — angular velocity in x, y, z (float, deg/s)

lat           — latitude in decimal degrees (float)

lon           — longitude in decimal degrees (float)

Write rate: 100 rows per second (driven by IMU sample rate)

GPS fields: updated at 1Hz, repeated across 100 rows

until next GPS fix arrives

Estimated storage: ~3.5MB per hour, ~28GB per day

Note: 8GB card provides approximately 2.3 days of

continuous logging at full rate. Duty cycling the

IMU (e.g. 5 min on / 55 min off) extends this to

approximately 27 days.

---

6. COMMUNICATIONS INTERFACE TO NETWORK SUBSYSTEM

--------------------------------------------------

Data handed to comms subsystem (LoRa beacon):

lat        — latitude (float, 4 bytes)

lon        — longitude (float, 4 bytes)

battery_%  — battery state of charge (uint8, 1 byte)

timestamp  — UTC timestamp (uint32, 4 bytes)

Total: 13 bytes per transmission

Transmission frequency: every 15 minutes

Justification:

- 15-minute interval consistent with minimum GPS

sampling rate validated by Alberello et al. (2020)

- Sufficient for real-time drift tracking without

excessive radio duty cycle

- LoRa beacon serves as liveness indicator and

coarse position update only — full data payload

resides on SD card for physical retrieval

---

7. DATA HANDLING PHILOSOPHY

-----------------------------

No onboard signal processing is performed.

Raw sensor data is written directly to SD card.

Rationale:

- Prior platforms (SHARC V3.0, LainePoiss) demonstrate

that onboard processing causes irrecoverable data loss

through decimation, fixed-length FFTs, and spectral

averaging (Noyce, 2024; Alari et al., 2022)

- Transmitting raw data preserves transient events

(e.g. floe-floe collisions) that spectral averaging

erases

- Post-processing on land via IMU-GPS fusion yields

well-resolved wave elevation estimates (Herbers et al., 2012)

- Embedded processor constraints on the STM32F411

preclude high-resolution FFT computation at 100Hz

sample rates without dedicated DSP hardware

The only firmware-level processing applied is:

- DC bias removal from accelerometer (single-pole

high-pass filter, cutoff ~0.01Hz) to remove

gravitational offset from heave channel

- NMEA sentence parsing for GPS (text processing only,

not signal processing)

---

8. KNOWN LIMITATIONS AND FUTURE WORK

--------------------------------------

- NEO-6M single constellation GPS degrades at polar

latitudes — replace with NEO-M9N for deployment

- No environmental sensing in current prototype —

add BME280 in next iteration

- No Extended Kalman Filter (EKF) implemented —

Robyn recommended EKF for IMU-GPS fusion; this is

identified as future work consistent with Herbers

et al. (2012) methodology

- SD card capacity limits continuous logging to ~2.3 days

at full rate — duty cycling or larger card recommended

- Magnetometer permanently disabled — no heading data

available; acceptable for wave measurement application

---

END OF DESIGN DECISIONS DOCUMENT